ОТЧЕТ

по лабораторной работе №2

по курсу «Инфокоммуникационные системы и сети»

на тему

«Основы адресации и маршрутизации в IP-сетях»

Выполнили:

магистранты группы 16ВВм1

Коновалов А.Ю.

Стекачев А.В.

Данилов В.Н.

Принял:

Коннов Н.Н.

Пенза 2018

Министерство образования Российской Федерации

Пензенский государственный университет

Кафедра «Вычислительная техника»

Цель работы

Изучение и практическое освоение основ адресации, разрешения физических адресов и простейшей маршрутизации в IP-сетях.

Лабораторное задание

Вариант 10. Файл со схемой сети: lab1_var10.jfst. Сеть между маршрутизаторами R1 и R2: 192.168.0.0. Компьютер PC1 имеет IP-адрес 172.168.0.1. Компьютер PC2 имеет IP-адрес 172.168.0.2. Компьютер PC4 имеет IP-адрес: 1.0.0.2.

Обозначения в задании: K1 – PC1, K2 – PC2, K3 – PC3.

С исходной моделью необходимо выполнить следующие действия:

- Исправить структуру сети (если это необходимо), обеспечив корректную доставку кадров на физическом уровне.

- Задать ip-адреса, маски подсети и шлюзы по умолчанию для всех узлов сети, чтобы обеспечить корректную доставку эхо-запроса от K1 к K2 и эхо-ответа обратно. Обосновать свои установки.

- Выполнить эхо-запрос с K1 на K2. Посмотреть вывод программы.

- Добавить статическую запись ARP для K3 на K1. Подождать устаревания ARP-таблиц и выполнить эхо запрос с K1 на K2. Объяснить результат.

- Выполнить эхо-запрос на IP-адрес 200.100.0.1 c K1. Объяснить вывод программы.

- Выполнить эхо запросы с K1 и K2 на все узлы сети. Убедиться, что эхо-ответы приходят.

Ход работы

- Задали ip-адреса, маски подсети и шлюзы по умолчанию для всех узлов сети так, как показано на рисунке 1.

В представленной сетевой модели было обнаружено 3 подсети:

- Подсеть 172.168.0.0: компьютеры PC1 и PC3. Маску подсети установили в 255.255.0.0.

- Подсеть 192.168.70.0: компьютер PC2, маршрутизатор R2. Маску подсети установили в 255.255.255.0.

- Подсеть 1.0.0.0: компьютер PC4 и маршрутизатор R3. Маску подсети установили в 255.0.0.0.

Рисунок 1

- Задали ip-адреса, маски подсети, шлюзы по умолчанию для всех компьютеров сети (табл. 1).

Таблица 1 – Сетевые настройки компьютеров

| Имя узла | IP-адрес | Маска сети | Шлюз по умолчанию |
| --- | --- | --- | --- |
| PC1 | 172.168.0.1 | 255.255.0.0 | 172.168.0.4 |
| PC2 | 192.168.70.1 | 255.255.255.0 | 192.168.70.2 |
| PC3 | 172.168.0.3 | 255.255.0.0 | 172.168.0.1 |
| PC4(BOSS) | 1.0.0.2 | 255.0.0.0 | 1.0.0.3 |

- Задали ip-адреса, маски подсети, таблицы маршрутизации для всех маршрутизаторов (табл. 2).

Таблица 2 – Сетевые настройки маршрутизаторов

| Имя узла | eth | IP-адрес | Маска сети | Шлюз по умолчанию |
| --- | --- | --- | --- | --- |
| R2 | eth0 | 172.168.0.4 | 255.255.255.0 | 192.168.70.1 |
|  | eth1 | 192.168.70.2 | 255.255.255.0 |  |
| R3 | eth0 | 172.168.0.2 | 255.255.0.0 | 1.0.0.2 |
|  | eth1 | 1.0.0.3 | 255.0.0.0 |  |

- Для роутера R2 настроили таблицу маршрутизации, в соответствии с рисунком 2

Рисунок 2

- Для роутера R3 настроили таблицу маршрутизации, в соответствии с рисунком 3

Рисунок 3

- Выполнили эхо-запрос с K1(PC1) на K2(PC2). Вывод программы:

| 00:44:27-895 | PC1 | Echo Request Packet | Network | Created Echo Request packet to 192.168.70.1 |
| --- | --- | --- | --- | --- |
| 00:44:27-896 | PC1 | ARP Discovery Packet | DataLink | Created ARP discovery packet to source MAC address for IP 172.168.0.4 |
| 00:44:27-896 | PC1 | ARP_packet | Network | Sending broadcast packet from ProtocolStack. |
| 00:44:27-896 | PC1 | Ethernet Packet | Link | Sending packet from interface 7D:6F:17:A3:25:2A |
| 00:44:27-896 | r3 | Ethernet Packet | Link | Recieved and accepted packet at interface 44:43:A8:64:53:30 |
| 00:44:27-897 | r3 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:44:27-897 | PC3 | Ethernet Packet | Link | Recieved and accepted packet at interface 2F:79:94:70:C1:6E |
| 00:44:27-897 | PC3 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:44:27-897 | R2 | Ethernet Packet | Link | Recieved and accepted packet at interface B0:3B:82:82:28:89 |
| 00:44:27-898 | R2 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:44:27-898 | R2 | ARP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:44:27-898 | R2 | ARP Response Packet | DataLink | Created ARP Response packet to 172.168.0.1 |
| 00:44:27-898 | R2 | ARP_packet | Network | Sending packet from ProtocolStack (to 172.168.0.1). |
| 00:44:27-899 | R2 | Ethernet Packet | Link | Sending packet from interface B0:3B:82:82:28:89 |
| 00:44:27-899 | r3 | Ethernet Packet | Link | Recieved and dropped packet at interface 44:43:A8:64:53:30 |
| 00:44:27-899 | PC3 | Ethernet Packet | Link | Recieved and dropped packet at interface 2F:79:94:70:C1:6E |
| 00:44:27-900 | PC1 | Ethernet Packet | Link | Recieved and accepted packet at interface 7D:6F:17:A3:25:2A |
| 00:44:27-900 | PC1 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:44:27-900 | PC1 | ARP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:44:27-900 | PC1 | ICMP_packet | Network | Sending packet from ProtocolStack (to 172.168.0.4). |
| 00:44:27-901 | PC1 | Ethernet Packet | Link | Sending packet from interface 7D:6F:17:A3:25:2A |
| 00:44:27-901 | r3 | Ethernet Packet | Link | Recieved and dropped packet at interface 44:43:A8:64:53:30 |
| 00:44:27-901 | PC3 | Ethernet Packet | Link | Recieved and dropped packet at interface 2F:79:94:70:C1:6E |
| 00:44:27-901 | R2 | Ethernet Packet | Link | Recieved and accepted packet at interface B0:3B:82:82:28:89 |
| 00:44:27-902 | R2 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:44:27-902 | R2 | ICMP_packet | Network | Packet Received: Network Layer Device is Routable forwarding packet. |
| 00:44:27-902 | R2 | ARP Discovery Packet | DataLink | Created ARP discovery packet to source MAC address for IP 192.168.70.1 |
| 00:44:27-903 | R2 | ARP_packet | Network | Sending broadcast packet from ProtocolStack. |
| 00:44:27-904 | R2 | Ethernet Packet | Link | Sending packet from interface 8C:60:7F:83:A2:26 |
| 00:44:27-904 | PC2 | Ethernet Packet | Link | Recieved and accepted packet at interface 44:2D:8D:93:2A:5D |
| 00:44:27-904 | PC2 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:44:27-904 | PC2 | ARP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:44:27-905 | PC2 | ARP Response Packet | DataLink | Created ARP Response packet to 192.168.70.2 |
| 00:44:27-905 | PC2 | ARP_packet | Network | Sending packet from ProtocolStack (to 192.168.70.2). |
| 00:44:27-905 | PC2 | Ethernet Packet | Link | Sending packet from interface 44:2D:8D:93:2A:5D |
| 00:44:27-906 | R2 | Ethernet Packet | Link | Recieved and accepted packet at interface 8C:60:7F:83:A2:26 |
| 00:44:27-906 | R2 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:44:27-906 | R2 | ARP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:44:27-906 | R2 | ICMP_packet | Network | Forwarding packet from ProtocolStack(to 192.168.70.1). |
| 00:44:27-907 | R2 | Ethernet Packet | Link | Sending packet from interface 8C:60:7F:83:A2:26 |
| 00:44:27-907 | PC2 | Ethernet Packet | Link | Recieved and accepted packet at interface 44:2D:8D:93:2A:5D |
| 00:44:27-907 | PC2 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:44:27-907 | PC2 | ICMP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:44:27-908 | PC2 | Echo Reply Packet | Network | Created Echo Reply packet to 172.168.0.1 |
| 00:44:27-908 | PC2 | ICMP_packet | Network | Sending packet from ProtocolStack (to 192.168.70.2). |
| 00:44:27-908 | PC2 | Ethernet Packet | Link | Sending packet from interface 44:2D:8D:93:2A:5D |
| 00:44:27-908 | R2 | Ethernet Packet | Link | Recieved and accepted packet at interface 8C:60:7F:83:A2:26 |
| 00:44:27-908 | R2 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:44:27-909 | R2 | ICMP_packet | Network | Packet Received: Network Layer Device is Routable forwarding packet. |
| 00:44:27-909 | R2 | ICMP_packet | Network | Forwarding packet from ProtocolStack(to 172.168.0.1). |
| 00:44:27-909 | R2 | Ethernet Packet | Link | Sending packet from interface B0:3B:82:82:28:89 |
| 00:44:27-909 | r3 | Ethernet Packet | Link | Recieved and dropped packet at interface 44:43:A8:64:53:30 |
| 00:44:27-910 | PC3 | Ethernet Packet | Link | Recieved and dropped packet at interface 2F:79:94:70:C1:6E |
| 00:44:27-910 | PC1 | Ethernet Packet | Link | Recieved and accepted packet at interface 7D:6F:17:A3:25:2A |
| 00:44:27-910 | PC1 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:44:27-910 | PC1 | ICMP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:44:27-911 | PC1 | Echo Reply Packet | Network | Echo reply packet received from 192.168.70.1 |

- После выполнения эхо-команды с PC1 на PC2 в ARP-таблице маршрутизатора R2 появились динамические записи для PC1 и PC2, как показано на рисунке 2.

Рисунок 4

До момента устаревания ARP-таблицы при выполнении последующих эхо-команд с BIG_BOSS на OFFICE1_pc4 ARP-запросы не посылались:

| 00:47:10-429 | PC1 | Echo Request Packet | Network | Created Echo Request packet to 192.168.70.1 |
| --- | --- | --- | --- | --- |
| 00:47:10-429 | PC1 | ICMP_packet | Network | Sending packet from ProtocolStack (to 172.168.0.4). |
| 00:47:10-429 | PC1 | Ethernet Packet | Link | Sending packet from interface 7D:6F:17:A3:25:2A |
| 00:47:10-430 | r3 | Ethernet Packet | Link | Recieved and dropped packet at interface 44:43:A8:64:53:30 |
| 00:47:10-430 | PC3 | Ethernet Packet | Link | Recieved and dropped packet at interface 2F:79:94:70:C1:6E |
| 00:47:10-430 | R2 | Ethernet Packet | Link | Recieved and accepted packet at interface B0:3B:82:82:28:89 |
| 00:47:10-430 | R2 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:47:10-430 | R2 | ICMP_packet | Network | Packet Received: Network Layer Device is Routable forwarding packet. |
| 00:47:10-430 | R2 | ICMP_packet | Network | Forwarding packet from ProtocolStack(to 192.168.70.1). |
| 00:47:10-430 | R2 | Ethernet Packet | Link | Sending packet from interface 8C:60:7F:83:A2:26 |
| 00:47:10-430 | PC2 | Ethernet Packet | Link | Recieved and accepted packet at interface 44:2D:8D:93:2A:5D |
| 00:47:10-430 | PC2 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:47:10-430 | PC2 | ICMP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:47:10-430 | PC2 | Echo Reply Packet | Network | Created Echo Reply packet to 172.168.0.1 |
| 00:47:10-430 | PC2 | ICMP_packet | Network | Sending packet from ProtocolStack (to 192.168.70.2). |
| 00:47:10-431 | PC2 | Ethernet Packet | Link | Sending packet from interface 44:2D:8D:93:2A:5D |
| 00:47:10-431 | R2 | Ethernet Packet | Link | Recieved and accepted packet at interface 8C:60:7F:83:A2:26 |
| 00:47:10-431 | R2 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:47:10-431 | R2 | ICMP_packet | Network | Packet Received: Network Layer Device is Routable forwarding packet. |
| 00:47:10-431 | R2 | ICMP_packet | Network | Forwarding packet from ProtocolStack(to 172.168.0.1). |
| 00:47:10-431 | R2 | Ethernet Packet | Link | Sending packet from interface B0:3B:82:82:28:89 |
| 00:47:10-431 | r3 | Ethernet Packet | Link | Recieved and dropped packet at interface 44:43:A8:64:53:30 |
| 00:47:10-431 | PC3 | Ethernet Packet | Link | Recieved and dropped packet at interface 2F:79:94:70:C1:6E |
| 00:47:10-431 | PC1 | Ethernet Packet | Link | Recieved and accepted packet at interface 7D:6F:17:A3:25:2A |
| 00:47:10-431 | PC1 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:47:10-431 | PC1 | ICMP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:47:10-431 | PC1 | Echo Reply Packet | Network | Echo reply packet received from 192.168.70.1 |

- Выполнили эхо-запрос на несуществующий узел с IP-адресом 200.199.125.1 c K1(PC1). Вывод программы:

| 00:50:31-753 | PC1 | Echo Request Packet | Network | Created Echo Request packet to 200.199.125.2 |
| --- | --- | --- | --- | --- |
| 00:50:31-753 | PC1 | ICMP_packet | Network | Sending packet from ProtocolStack (to 172.168.0.4). |
| 00:50:31-753 | PC1 | Ethernet Packet | Link | Sending packet from interface 7D:6F:17:A3:25:2A |
| 00:50:31-753 | r3 | Ethernet Packet | Link | Recieved and dropped packet at interface 44:43:A8:64:53:30 |
| 00:50:31-754 | PC3 | Ethernet Packet | Link | Recieved and dropped packet at interface 2F:79:94:70:C1:6E |
| 00:50:31-754 | R2 | Ethernet Packet | Link | Recieved and accepted packet at interface B0:3B:82:82:28:89 |
| 00:50:31-754 | R2 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:50:31-754 | R2 | ICMP_packet | Network | Packet Received: Network Layer Device is Routable forwarding packet. |
| 00:50:31-754 | R2 | ICMP_packet | Network | Forwarding packet from ProtocolStack(to 192.168.70.1). |
| 00:50:31-755 | R2 | Ethernet Packet | Link | Sending packet from interface 8C:60:7F:83:A2:26 |
| 00:50:31-755 | PC2 | Ethernet Packet | Link | Recieved and accepted packet at interface 44:2D:8D:93:2A:5D |
| 00:50:31-755 | PC2 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |

Как видно, пакет попал в петлю между двумя маршрутизаторами R2 и PC2 и находился там, пока его время жизни не истекло.

- Выполнили эхо запросы с K1(PC1) и K2(PC2) на все узлы сети. Убедились, что эхо-ответы приходят.

Вывод

В процессе выполнения лабораторного задания изучили основы адресации, разрешения физических адресов и простейшей маршрутизации в IP-сетях.