ОТЧЕТ

по лабораторной работе №3

по курсу «Инфокоммуникационные системы и сети»

на тему

«Основы статической маршрутизации в IP-сетях»

Выполнили:

магистранты группы 16ВВм1

Коновалов А.Ю.

Стекачев А.В.

Данилов В.Н.

Принял:

Коннов Н.Н.

Пенза 2018

Министерство образования Российской Федерации

Пензенский государственный университет

Кафедра «Вычислительная техника»

Цель работы

Изучить методы статической маршрутизации в IP-сетях, научиться управлять таблицами маршрутизации на узлах сетевого уровня.

Лабораторное задание

Вариант 7. Файл со схемой сети: lab2_var7.jfst. Сеть между узлами PC3 и R3, R4, R6: 204.188.45.128. Сеть между маршрутизаторами R1, R2, R3: 204.188.45.192. Компьютер PC1 имеет IP-адрес 204.188.45.1. Компьютер PC2 имеет IP-адрес 204.188.45.65. Компьютер PC3 имеет IPадрес 204.188.45.129. Компьютер BOSS имеет IP-адрес 204.188.45.196.

Обозначения в задании: K1 – PC1, K2 – BOSS, K3 – PC3.

С исходной моделью необходимо выполнить следующие действия:

- Для всех узлов сети установить IP-адреса, маски подсетей и шлюзы по умолчанию, чтобы добиться успешного выполнения эхо-запроса ближайших соседей (находящихся в одной подсети).

- Настроить таблицы маршрутизации на маршрутизаторах, чтобы добиться доставки пакетов от узла K1 к узлу K2 и обратно, от узла K2 к K3 и обратно, от узла K3 к K1 и обратно. Пакеты должны доходить до узлов кратчайшим путем.

- Настроить таблицы маршрутизации на узлах K1, K2 и K3 чтобы обеспечить кратчайшую доставку пакетов между этими узлами, если это невозможно было обеспечить в пункте 2.

Ход работы

- В представленной сетевой модели было обнаружено 4 подсети:

- Подсеть 204.188.46.0: компьютер PC1, маршрутизаторы R1, R4, R5. Маску подсети установили в 255.255.255.0.

- Подсеть 204.188.48.0: компьютер PC2, маршрутизаторы R2, R5, R6. Маску подсети установили в 255.255.255.0.

- Подсеть 204.188.45.0: компьютер PC3 маршрутизаторы R4, R6, R3. Маску подсети установили в 255.255.255.0.

- Подсеть 204.188.47.0: компьютер BOSS, маршрутизаторы R1, R2, R3. Маску подсети установили в 255.255.255.0.

Рисунок 1

- Задали ip-адреса, маски подсети, шлюзы по умолчанию для всех компьютеров сети (табл. 1).

Таблица 1 – Сетевые настройки компьютеров

| Имя узла | IP-адрес | Маска сети | Шлюз по умолчанию |
| --- | --- | --- | --- |
| PC1 | 204.188.46.1 | 255.255.255.0 | 204.188.46.2 |
| PC2 | 204.188.48.65 | 255.255.255.0 | 204.188.48.66 |
| PC3 | 204.188.45.129 | 255.255.255.0 | 204.188.45.195 |
| PC4(BOSS) | 204.188.47.196 | 255.255.255.0 | 204.188.47.193 |

- Задали ip-адреса, маски подсети, таблицы маршрутизации для всех маршрутизаторов (табл. 2).

Таблица 2 – Сетевые настройки маршрутизаторов

| Имя узла | eth | IP-адрес | Маска сети |
| --- | --- | --- | --- |
| R1 | eth0 | 204.188.46.2 | 255.255.255.0 |
|  | eth1 | 204.188.47.193 | 255.255.255.0 |
| R2 | eth0 | 204.188.48.66 | 255.255.255.0 |
|  | eth1 | 204.188.47.194 | 255.255.255.0 |
| R3 | eth0 | 204.188.45.195 | 255.255.255.0 |
|  | eth1 | 204.188.47.130 | 255.255.255.0 |
| R4 | eth0 | 204.188.46.3 | 255.255.255.0 |
|  | eth1 | 204.188.45.131 | 255.255.255.0 |
| R5 | eth0 | 204.188.46.67 | 255.255.255.0 |
|  | eth1 | 204.188.48.4 | 255.255.255.0 |
| R6 | eth0 | 204.188.45.68 | 255.255.255.0 |
|  | eth1 | 204.188.45.132 | 255.255.255.0 |

- Выполнили настройку шлюзов по умолчанию, а также добавили статические записи в таблицы маршрутизации для роутеров.

- Для роутера R1 настроили таблицу маршрутизации, в соответствии с рисунком 2.

Рисунок 2 Роутер R1

- Для роутера R2 настроили таблицу маршрутизации, в соответствии с рисунком 3.

Рисунок 3 Роутер R2

- Для роутера R3 настроили таблицу маршрутизации, в соответствии с рисунком 4.

Рисунок 4 Роутер R3

- Выполнили эхо-запрос с K1(PC1) на K2(BOSS). Вывод программы (ARP-пакеты не представлены):

| 00:00:28-169 | PC1 | ARP_packet | Network | Sending broadcast packet from ProtocolStack. |
| --- | --- | --- | --- | --- |
| 00:00:28-169 | PC1 | Ethernet Packet | Link | Sending packet from interface A3:69:7D:C3:45:3E |
| 00:00:28-169 | R4 | Ethernet Packet | Link | Recieved and accepted packet at interface 5A:6B:AA:65:7F:8C |
| 00:00:28-169 | R4 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:00:28-169 | R5 | Ethernet Packet | Link | Recieved and accepted packet at interface C7:52:B5:9A:4D:2F |
| 00:00:28-169 | R5 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:00:28-169 | R1 | Ethernet Packet | Link | Recieved and accepted packet at interface C5:8F:C4:60:B9:95 |
| 00:00:28-169 | R1 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:00:28-169 | R1 | ARP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:00:28-169 | R1 | ARP Response Packet | DataLink | Created ARP Response packet to 204.188.46.1 |
| 00:00:28-169 | R1 | ARP_packet | Network | Sending packet from ProtocolStack (to 204.188.46.1). |
| 00:00:28-169 | R1 | Ethernet Packet | Link | Sending packet from interface C5:8F:C4:60:B9:95 |
| 00:00:28-169 | R4 | Ethernet Packet | Link | Recieved and dropped packet at interface 5A:6B:AA:65:7F:8C |
| 00:00:28-169 | R5 | Ethernet Packet | Link | Recieved and dropped packet at interface C7:52:B5:9A:4D:2F |
| 00:00:28-169 | PC1 | Ethernet Packet | Link | Recieved and accepted packet at interface A3:69:7D:C3:45:3E |
| 00:00:28-169 | PC1 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:00:28-169 | PC1 | ARP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:00:28-169 | PC1 | ICMP_packet | Network | Sending packet from ProtocolStack (to 204.188.46.2). |
| 00:00:28-169 | PC1 | Ethernet Packet | Link | Sending packet from interface A3:69:7D:C3:45:3E |
| 00:00:28-169 | R4 | Ethernet Packet | Link | Recieved and dropped packet at interface 5A:6B:AA:65:7F:8C |
| 00:00:28-169 | R5 | Ethernet Packet | Link | Recieved and dropped packet at interface C7:52:B5:9A:4D:2F |
| 00:00:28-169 | R1 | Ethernet Packet | Link | Recieved and accepted packet at interface C5:8F:C4:60:B9:95 |
| 00:00:28-169 | R1 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:00:28-169 | R1 | ICMP_packet | Network | Packet Received: Network Layer Device is Routable forwarding packet. |
| 00:00:28-169 | R1 | ARP Discovery Packet | DataLink | Created ARP discovery packet to source MAC address for IP 204.188.47.196 |
| 00:00:28-169 | R1 | ARP_packet | Network | Sending broadcast packet from ProtocolStack. |
| 00:00:28-169 | R1 | Ethernet Packet | Link | Sending packet from interface 55:A4:34:B4:8E:9B |
| 00:00:28-169 | BOSS | Ethernet Packet | Link | Recieved and accepted packet at interface 15:8B:3D:15:4B:24 |
| 00:00:28-169 | BOSS | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:00:28-169 | BOSS | ARP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:00:28-169 | BOSS | ARP Response Packet | DataLink | Created ARP Response packet to 204.188.47.193 |
| 00:00:28-169 | BOSS | ARP_packet | Network | Sending packet from ProtocolStack (to 204.188.47.193). |
| 00:00:28-169 | BOSS | Ethernet Packet | Link | Sending packet from interface 15:8B:3D:15:4B:24 |
| 00:00:28-169 | R3 | Ethernet Packet | Link | Recieved and dropped packet at interface 3B:57:C0:62:90:90 |
| 00:00:28-169 | R2 | Ethernet Packet | Link | Recieved and dropped packet at interface 3A:AA:C3:4E:B6:59 |
| 00:00:28-169 | R1 | Ethernet Packet | Link | Recieved and accepted packet at interface 55:A4:34:B4:8E:9B |
| 00:00:28-169 | R1 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:00:28-169 | R1 | ARP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:00:28-169 | R3 | Ethernet Packet | Link | Recieved and accepted packet at interface 3B:57:C0:62:90:90 |
| 00:00:28-169 | R3 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:00:28-169 | R2 | Ethernet Packet | Link | Recieved and accepted packet at interface 3A:AA:C3:4E:B6:59 |
| 00:00:28-169 | R2 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:00:28-169 | R1 | ICMP_packet | Network | Forwarding packet from ProtocolStack(to 204.188.47.196). |
| 00:00:28-169 | R1 | Ethernet Packet | Link | Sending packet from interface 55:A4:34:B4:8E:9B |
| 00:00:28-169 | BOSS | Ethernet Packet | Link | Recieved and accepted packet at interface 15:8B:3D:15:4B:24 |
| 00:00:28-169 | BOSS | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:00:28-169 | BOSS | ICMP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:00:28-169 | BOSS | Echo Reply Packet | Network | Created Echo Reply packet to 204.188.46.1 |
| 00:00:28-169 | BOSS | ICMP_packet | Network | Sending packet from ProtocolStack (to 204.188.47.193). |
| 00:00:28-169 | BOSS | Ethernet Packet | Link | Sending packet from interface 15:8B:3D:15:4B:24 |
| 00:00:28-169 | R3 | Ethernet Packet | Link | Recieved and dropped packet at interface 3B:57:C0:62:90:90 |
| 00:00:28-169 | R2 | Ethernet Packet | Link | Recieved and dropped packet at interface 3A:AA:C3:4E:B6:59 |
| 00:00:28-169 | R1 | Ethernet Packet | Link | Recieved and accepted packet at interface 55:A4:34:B4:8E:9B |
| 00:00:28-169 | R1 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:00:28-169 | R1 | ICMP_packet | Network | Packet Received: Network Layer Device is Routable forwarding packet. |
| 00:00:28-169 | R1 | ICMP_packet | Network | Forwarding packet from ProtocolStack(to 204.188.46.1). |
| 00:00:28-169 | R1 | Ethernet Packet | Link | Sending packet from interface C5:8F:C4:60:B9:95 |
| 00:00:28-169 | R4 | Ethernet Packet | Link | Recieved and dropped packet at interface 5A:6B:AA:65:7F:8C |
| 00:00:28-169 | R5 | Ethernet Packet | Link | Recieved and dropped packet at interface C7:52:B5:9A:4D:2F |
| 00:00:28-169 | PC1 | Ethernet Packet | Link | Recieved and accepted packet at interface A3:69:7D:C3:45:3E |
| 00:00:28-169 | PC1 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:00:28-169 | PC1 | ICMP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:00:28-169 | PC1 | Echo Reply Packet | Network | Echo reply packet received from 204.188.47.196 |
| 00:00:28-169 | R3 | Ethernet Packet | Link | Recieved and dropped packet at interface 3B:57:C0:62:90:90 |
| 00:00:28-169 | R2 | Ethernet Packet | Link | Recieved and dropped packet at interface 3A:AA:C3:4E:B6:59 |

Из результатов эхо-запроса видно, что ICMP-пакет проходит по кратчайшему маршруту туда и обратно.

- Выполнили эхо-запрос в обратном направлении − с K2(BOSS) на K1(PC1). Убедились, что ICMP-пакет проходит по кратчайшему маршруту.

- Выполнили эхо-запрос с K2(BOSS) на K3(PC3). Вывод программы (ARP-пакеты не представлены):

| 00:01:30-156 | BOSS | Echo Request Packet | Network | Created Echo Request packet to 204.188.45.129 |
| --- | --- | --- | --- | --- |
| 00:01:30-156 | BOSS | ICMP_packet | Network | Sending packet from ProtocolStack (to 204.188.47.193). |
| 00:01:30-156 | BOSS | Ethernet Packet | Link | Sending packet from interface 15:8B:3D:15:4B:24 |
| 00:01:30-156 | R3 | Ethernet Packet | Link | Recieved and dropped packet at interface 3B:57:C0:62:90:90 |
| 00:01:30-156 | R2 | Ethernet Packet | Link | Recieved and dropped packet at interface 3A:AA:C3:4E:B6:59 |
| 00:01:30-156 | R1 | Ethernet Packet | Link | Recieved and accepted packet at interface 55:A4:34:B4:8E:9B |
| 00:01:30-156 | R1 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:01:30-156 | R1 | ICMP_packet | Network | Packet Received: Network Layer Device is Routable forwarding packet. |
| 00:01:30-156 | R1 | ARP Discovery Packet | DataLink | Created ARP discovery packet to source MAC address for IP 204.188.47.130 |
| 00:01:30-156 | R1 | ARP_packet | Network | Sending broadcast packet from ProtocolStack. |
| 00:01:30-156 | R1 | Ethernet Packet | Link | Sending packet from interface 55:A4:34:B4:8E:9B |
| 00:01:30-156 | BOSS | Ethernet Packet | Link | Recieved and accepted packet at interface 15:8B:3D:15:4B:24 |
| 00:01:30-156 | BOSS | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:01:30-156 | R3 | Ethernet Packet | Link | Recieved and accepted packet at interface 3B:57:C0:62:90:90 |
| 00:01:30-156 | R3 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:01:30-156 | R3 | ARP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:01:30-156 | R3 | ARP Response Packet | DataLink | Created ARP Response packet to 204.188.47.193 |
| 00:01:30-156 | R3 | ARP_packet | Network | Sending packet from ProtocolStack (to 204.188.47.193). |
| 00:01:30-156 | R3 | Ethernet Packet | Link | Sending packet from interface 3B:57:C0:62:90:90 |
| 00:01:30-156 | BOSS | Ethernet Packet | Link | Recieved and dropped packet at interface 15:8B:3D:15:4B:24 |
| 00:01:30-156 | R2 | Ethernet Packet | Link | Recieved and dropped packet at interface 3A:AA:C3:4E:B6:59 |
| 00:01:30-156 | R1 | Ethernet Packet | Link | Recieved and accepted packet at interface 55:A4:34:B4:8E:9B |
| 00:01:30-156 | R1 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:01:30-156 | R1 | ARP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:01:30-156 | R2 | Ethernet Packet | Link | Recieved and accepted packet at interface 3A:AA:C3:4E:B6:59 |
| 00:01:30-156 | R2 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:01:30-156 | R1 | ICMP_packet | Network | Forwarding packet from ProtocolStack(to 204.188.47.130). |
| 00:01:30-172 | R1 | Ethernet Packet | Link | Sending packet from interface 55:A4:34:B4:8E:9B |
| 00:01:30-172 | BOSS | Ethernet Packet | Link | Recieved and dropped packet at interface 15:8B:3D:15:4B:24 |
| 00:01:30-172 | R3 | Ethernet Packet | Link | Recieved and accepted packet at interface 3B:57:C0:62:90:90 |
| 00:01:30-172 | R3 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:01:30-172 | R3 | ICMP_packet | Network | Packet Received: Network Layer Device is Routable forwarding packet. |
| 00:01:30-172 | R3 | ARP Discovery Packet | DataLink | Created ARP discovery packet to source MAC address for IP 204.188.45.129 |
| 00:01:30-172 | R3 | ARP_packet | Network | Sending broadcast packet from ProtocolStack. |
| 00:01:30-172 | R3 | Ethernet Packet | Link | Sending packet from interface 6D:B1:53:9C:29:64 |
| 00:01:30-172 | R4 | Ethernet Packet | Link | Recieved and accepted packet at interface 56:7A:54:89:1F:B4 |
| 00:01:30-172 | R4 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:01:30-172 | R6 | Ethernet Packet | Link | Recieved and accepted packet at interface 36:2D:8F:3F:A6:9C |
| 00:01:30-172 | R6 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:01:30-172 | PC3 | Ethernet Packet | Link | Recieved and accepted packet at interface 4F:B1:79:AA:49:1E |
| 00:01:30-172 | PC3 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:01:30-172 | PC3 | ARP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:01:30-172 | PC3 | ARP Response Packet | DataLink | Created ARP Response packet to 204.188.45.195 |
| 00:01:30-172 | PC3 | ARP_packet | Network | Sending packet from ProtocolStack (to 204.188.45.195). |
| 00:01:30-172 | PC3 | Ethernet Packet | Link | Sending packet from interface 4F:B1:79:AA:49:1E |
| 00:01:30-172 | R4 | Ethernet Packet | Link | Recieved and dropped packet at interface 56:7A:54:89:1F:B4 |
| 00:01:30-172 | R6 | Ethernet Packet | Link | Recieved and dropped packet at interface 36:2D:8F:3F:A6:9C |
| 00:01:30-172 | R3 | Ethernet Packet | Link | Recieved and accepted packet at interface 6D:B1:53:9C:29:64 |
| 00:01:30-172 | R3 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:01:30-172 | R3 | ARP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:01:30-172 | R3 | ICMP_packet | Network | Forwarding packet from ProtocolStack(to 204.188.45.129). |
| 00:01:30-172 | R3 | Ethernet Packet | Link | Sending packet from interface 6D:B1:53:9C:29:64 |
| 00:01:30-172 | R4 | Ethernet Packet | Link | Recieved and dropped packet at interface 56:7A:54:89:1F:B4 |
| 00:01:30-172 | R6 | Ethernet Packet | Link | Recieved and dropped packet at interface 36:2D:8F:3F:A6:9C |
| 00:01:30-172 | PC3 | Ethernet Packet | Link | Recieved and accepted packet at interface 4F:B1:79:AA:49:1E |
| 00:01:30-172 | PC3 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:01:30-172 | PC3 | ICMP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:01:30-172 | PC3 | Echo Reply Packet | Network | Created Echo Reply packet to 204.188.47.196 |
| 00:01:30-172 | PC3 | ICMP_packet | Network | Sending packet from ProtocolStack (to 204.188.45.195). |
| 00:01:30-172 | PC3 | Ethernet Packet | Link | Sending packet from interface 4F:B1:79:AA:49:1E |
| 00:01:30-172 | R4 | Ethernet Packet | Link | Recieved and dropped packet at interface 56:7A:54:89:1F:B4 |
| 00:01:30-172 | R6 | Ethernet Packet | Link | Recieved and dropped packet at interface 36:2D:8F:3F:A6:9C |
| 00:01:30-172 | R3 | Ethernet Packet | Link | Recieved and accepted packet at interface 6D:B1:53:9C:29:64 |
| 00:01:30-172 | R3 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:01:30-172 | R3 | ICMP_packet | Network | Packet Received: Network Layer Device is Routable forwarding packet. |
| 00:01:30-172 | R3 | ARP Discovery Packet | DataLink | Created ARP discovery packet to source MAC address for IP 204.188.47.196 |
| 00:01:30-172 | R3 | ARP_packet | Link | Sending broadcast packet from ProtocolStack. |
| 00:01:30-172 | R3 | Ethernet Packet | Link | Sending packet from interface 3B:57:C0:62:90:90 |
| 00:01:30-172 | BOSS | Ethernet Packet | Link | Recieved and accepted packet at interface 15:8B:3D:15:4B:24 |
| 00:01:30-172 | BOSS | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:01:30-172 | BOSS | ARP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:01:30-172 | BOSS | ARP Response Packet | DataLink | Created ARP Response packet to 204.188.47.130 |
| 00:01:30-172 | BOSS | ARP_packet | Network | Sending packet from ProtocolStack (to 204.188.47.130). |
| 00:01:30-172 | BOSS | Ethernet Packet | Link | Sending packet from interface 15:8B:3D:15:4B:24 |
| 00:01:30-172 | R3 | Ethernet Packet | Link | Recieved and accepted packet at interface 3B:57:C0:62:90:90 |
| 00:01:30-172 | R3 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:01:30-172 | R3 | ARP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:01:30-172 | R2 | Ethernet Packet | Link | Recieved and dropped packet at interface 3A:AA:C3:4E:B6:59 |
| 00:01:30-172 | R1 | Ethernet Packet | Link | Recieved and dropped packet at interface 55:A4:34:B4:8E:9B |
| 00:01:30-172 | R2 | Ethernet Packet | Link | Recieved and accepted packet at interface 3A:AA:C3:4E:B6:59 |
| 00:01:30-172 | R2 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:01:30-172 | R1 | Ethernet Packet | Link | Recieved and accepted packet at interface 55:A4:34:B4:8E:9B |
| 00:01:30-172 | R1 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:01:30-172 | R3 | ICMP_packet | Network | Forwarding packet from ProtocolStack(to 204.188.47.196). |
| 00:01:30-172 | R3 | Ethernet Packet | Link | Sending packet from interface 3B:57:C0:62:90:90 |
| 00:01:30-172 | BOSS | Ethernet Packet | Link | Recieved and accepted packet at interface 15:8B:3D:15:4B:24 |
| 00:01:30-172 | BOSS | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:01:30-172 | BOSS | ICMP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:01:30-172 | BOSS | Echo Reply Packet | Network | Echo reply packet received from 204.188.45.129 |
| 00:01:30-172 | R2 | Ethernet Packet | Link | Recieved and dropped packet at interface 3A:AA:C3:4E:B6:59 |
| 00:01:30-172 | R1 | Ethernet Packet | Link | Recieved and dropped packet at interface 55:A4:34:B4:8E:9B |
| 00:01:30-172 | R2 | Ethernet Packet | Link | Recieved and dropped packet at interface 3A:AA:C3:4E:B6:59 |

Из результатов эхо-запроса видно, что ICMP-пакет проходит по кратчайшему маршруту туда и обратно.

- Выполнили эхо-запрос в обратном направлении − с K3(PC3) на K2(BOSS). Убедились, что ICMP-пакет проходит по кратчайшему маршруту.

- Выполнили эхо-запрос с K3(PC3) на K1(PC1). Вывод программы (ARP-пакеты не представлены):

| 00:02:44-108 | PC3 | Echo Request Packet | Network | Created Echo Request packet to 204.188.46.1 |
| --- | --- | --- | --- | --- |
| 00:02:44-108 | PC3 | ICMP_packet | Network | Sending packet from ProtocolStack (to 204.188.45.195). |
| 00:02:44-108 | PC3 | Ethernet Packet | Link | Sending packet from interface 4F:B1:79:AA:49:1E |
| 00:02:44-108 | R4 | Ethernet Packet | Link | Recieved and dropped packet at interface 56:7A:54:89:1F:B4 |
| 00:02:44-108 | R6 | Ethernet Packet | Link | Recieved and dropped packet at interface 36:2D:8F:3F:A6:9C |
| 00:02:44-108 | R3 | Ethernet Packet | Link | Recieved and accepted packet at interface 6D:B1:53:9C:29:64 |
| 00:02:44-108 | R3 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:02:44-108 | R3 | ICMP_packet | Network | Packet Received: Network Layer Device is Routable forwarding packet. |
| 00:02:44-108 | R3 | ICMP_packet | Network | Forwarding packet from ProtocolStack(to 204.188.47.193). |
| 00:02:44-108 | R3 | Ethernet Packet | Link | Sending packet from interface 3B:57:C0:62:90:90 |
| 00:02:44-108 | BOSS | Ethernet Packet | Link | Recieved and dropped packet at interface 15:8B:3D:15:4B:24 |
| 00:02:44-108 | R2 | Ethernet Packet | Link | Recieved and dropped packet at interface 3A:AA:C3:4E:B6:59 |
| 00:02:44-108 | R1 | Ethernet Packet | Link | Recieved and accepted packet at interface 55:A4:34:B4:8E:9B |
| 00:02:44-108 | R1 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:02:44-108 | R1 | ICMP_packet | Network | Packet Received: Network Layer Device is Routable forwarding packet. |
| 00:02:44-108 | R1 | ARP Discovery Packet | DataLink | Created ARP discovery packet to source MAC address for IP 204.188.46.1 |
| 00:02:44-108 | R1 | ARP_packet | Network | Sending broadcast packet from ProtocolStack. |
| 00:02:44-108 | R1 | Ethernet Packet | Link | Sending packet from interface C5:8F:C4:60:B9:95 |
| 00:02:44-108 | R4 | Ethernet Packet | Link | Recieved and accepted packet at interface 5A:6B:AA:65:7F:8C |
| 00:02:44-108 | R4 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:02:44-108 | R5 | Ethernet Packet | Link | Recieved and accepted packet at interface C7:52:B5:9A:4D:2F |
| 00:02:44-108 | R5 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:02:44-108 | PC1 | Ethernet Packet | Link | Recieved and accepted packet at interface A3:69:7D:C3:45:3E |
| 00:02:44-108 | PC1 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:02:44-108 | PC1 | ARP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:02:44-108 | PC1 | ARP Response Packet | DataLink | Created ARP Response packet to 204.188.46.2 |
| 00:02:44-108 | PC1 | ARP_packet | Network | Sending packet from ProtocolStack (to 204.188.46.2). |
| 00:02:44-108 | PC1 | Ethernet Packet | Link | Sending packet from interface A3:69:7D:C3:45:3E |
| 00:02:44-108 | R4 | Ethernet Packet | Link | Recieved and dropped packet at interface 5A:6B:AA:65:7F:8C |
| 00:02:44-108 | R5 | Ethernet Packet | Link | Recieved and dropped packet at interface C7:52:B5:9A:4D:2F |
| 00:02:44-108 | R1 | Ethernet Packet | Link | Recieved and accepted packet at interface C5:8F:C4:60:B9:95 |
| 00:02:44-108 | R1 | ARP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:02:44-108 | R1 | ARP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:02:44-108 | R1 | ICMP_packet | Network | Forwarding packet from ProtocolStack(to 204.188.46.1). |
| 00:02:44-108 | R1 | Ethernet Packet | Link | Sending packet from interface C5:8F:C4:60:B9:95 |
| 00:02:44-108 | R4 | Ethernet Packet | Link | Recieved and dropped packet at interface 5A:6B:AA:65:7F:8C |
| 00:02:44-108 | R5 | Ethernet Packet | Link | Recieved and dropped packet at interface C7:52:B5:9A:4D:2F |
| 00:02:44-108 | PC1 | Ethernet Packet | Link | Recieved and accepted packet at interface A3:69:7D:C3:45:3E |
| 00:02:44-108 | PC1 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:02:44-108 | PC1 | ICMP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:02:44-108 | PC1 | Echo Reply Packet | Network | Created Echo Reply packet to 204.188.45.129 |
| 00:02:44-108 | PC1 | ICMP_packet | Network | Sending packet from ProtocolStack (to 204.188.46.2). |
| 00:02:44-108 | PC1 | Ethernet Packet | Link | Sending packet from interface A3:69:7D:C3:45:3E |
| 00:02:44-108 | R4 | Ethernet Packet | Link | Recieved and dropped packet at interface 5A:6B:AA:65:7F:8C |
| 00:02:44-108 | R5 | Ethernet Packet | Link | Recieved and dropped packet at interface C7:52:B5:9A:4D:2F |
| 00:02:44-108 | R1 | Ethernet Packet | Link | Recieved and accepted packet at interface C5:8F:C4:60:B9:95 |
| 00:02:44-108 | R1 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:02:44-108 | R1 | ICMP_packet | Network | Packet Received: Network Layer Device is Routable forwarding packet. |
| 00:02:44-108 | R1 | ICMP_packet | Network | Forwarding packet from ProtocolStack(to 204.188.47.130). |
| 00:02:44-108 | R1 | Ethernet Packet | Link | Sending packet from interface 55:A4:34:B4:8E:9B |
| 00:02:44-108 | BOSS | Ethernet Packet | Link | Recieved and dropped packet at interface 15:8B:3D:15:4B:24 |
| 00:02:44-108 | R3 | Ethernet Packet | Link | Recieved and accepted packet at interface 3B:57:C0:62:90:90 |
| 00:02:44-108 | R3 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:02:44-108 | R3 | ICMP_packet | Network | Packet Received: Network Layer Device is Routable forwarding packet. |
| 00:02:44-108 | R3 | ICMP_packet | Network | Forwarding packet from ProtocolStack(to 204.188.45.129). |
| 00:02:44-108 | R3 | Ethernet Packet | Link | Sending packet from interface 6D:B1:53:9C:29:64 |
| 00:02:44-108 | R4 | Ethernet Packet | Link | Recieved and dropped packet at interface 56:7A:54:89:1F:B4 |
| 00:02:44-108 | R6 | Ethernet Packet | Link | Recieved and dropped packet at interface 36:2D:8F:3F:A6:9C |
| 00:02:44-108 | PC3 | Ethernet Packet | Link | Recieved and accepted packet at interface 4F:B1:79:AA:49:1E |
| 00:02:44-108 | PC3 | ICMP_packet | Network | ProtocolStack received packet from local Interface. |
| 00:02:44-108 | PC3 | ICMP_packet | Network | Confirmed Packet is for this Network Layer Device. |
| 00:02:44-108 | PC3 | Echo Reply Packet | Network | Echo reply packet received from 204.188.46.1 |
| 00:02:44-108 | R2 | Ethernet Packet | Link | Recieved and dropped packet at interface 3A:AA:C3:4E:B6:59 |

Из результатов эхо-запроса видно, что ICMP-пакет проходит по кратчайшему маршруту.

- Выполнили эхо-запрос в обратном направлении − с K1(PC1) на K3(PC3). Убедились, что ICMP-пакет проходит по кратчайшему маршруту.

Вывод

В ходе выполнения лабораторной работы изучили методы статической маршрутизации в IP-сетях, научились управлять таблицами маршрутизации на узлах сетевого уровня.