Министерство науки и высшего образования Российской Федерации

Пензенский государственный университет

Кафедра «Вычислительная техника»

ОТЧЁТ

по лабораторной работе № 8

по курсу «Проектирование программного обеспечения микропроцессорных систем»

на тему «Ввод-вывод данных через последовательный порт»

Вариант №7

Выполнили:

Студенты группы 22ВВП1

Беляев Д. И.

Демин М. С.

Сергунов М. Р.

Приняли:

Бычков А. С.

Никишин К.И.

Пенза 2025

Цель работы

Изучение методов ввода-вывода данных через последовательный порт.

Ход работы

Разработали программу, осуществляющую ввод-вывод данных через последовательный порт.

Листинг программы

;===============================================================================

; UART INITIALIZATION (MODE 1, 8-BIT UART, VARIABLE BAUD RATE)

;===============================================================================

CLR     SM0                ; SM0=0, SM1=1 → UART Mode 1 (8-bit UART)

SETB    SM1                ; 1 start bit, 8 data bits, 1 stop bit

;-------------------------------------------------------------------------------

; Enable double baud rate (PCON.7 = SMOD = 1)

;-------------------------------------------------------------------------------

MOV     A, PCON            ; Read PCON so we don’t overwrite other bits

SETB    ACC.7              ; Set SMOD = 1 → doubles UART baud rate

MOV     PCON, A            ; Write back updated value to PCON

;-------------------------------------------------------------------------------

; Enable UART receiver

;-------------------------------------------------------------------------------

SETB    REN                ; REN=1 → allow receiving bytes via UART

;===============================================================================

; TIMER1 CONFIGURATION FOR UART BAUD RATE

; Timer1 Mode 2 = 8-bit auto-reload (standard for UART)

; TH1 = TL1 = 243 → reload every 13 counts (baud depends on SMOD & clock)

;===============================================================================

MOV     TMOD, #20H         ; Timer1 Mode 2 (auto-reload), Timer0 unchanged

MOV     TH1,  #243         ; Reload value (256 - 13) = 243

MOV     TL1,  #243         ; Load initial counter value

SETB    TR1                ; Start Timer1 (TCON.TR1=1)

;===============================================================================

; RECEIVE PHASE INITIALIZATION

; R1 → write pointer for received data buffer

; R0 → remaining number of bytes to receive

;===============================================================================

MOV     R1, #47H           ; Start address of receive buffer

MOV     R0, #0AH           ; Expecting 10 bytes

;===============================================================================

; RECEIVE LOOP

; Each received byte may have parity information stored in bit7.

;===============================================================================

RECEIVE_BYTE:

JNB     RI, $              ; Wait until hardware sets RI (byte received)

CLR     RI                 ; Must clear RI before next reception

MOV     A, SBUF            ; Read received byte from UART buffer

;-------------------------------------------------------------

; PARITY HANDLING — MSB (ACC.7) carries parity bit

;

; If ACC.7 = 0 → no parity bit present → skip processing.

; If ACC.7 = 1 → parity bit included:

;     - Clear ACC.7 to restore the original 7/8 data bits

;     - Use flag P to determine actual parity of data

;     - Store parity mode flags in register B:

;           B.4 = 1 → parity enabled

;           B.0 = 0 or 1 → even/odd behavior

;-------------------------------------------------------------

JNB     ACC.7, STORE_BYTE  ; If bit7=0 → no parity included → skip checks

CLR     ACC.7              ; Remove parity bit from the received value

; (This also updates CPU parity flag P)

JNB     P, PARITY_EVEN     ; If P=0 → even number of 1s → parity is EVEN

;----------- ODD parity case ---------------

MOV     B, #10H            ; B = 00010000b → parity enabled, odd mode

SJMP    STORE_BYTE

PARITY_EVEN:

MOV     B, #11H            ; B = 00010001b → parity enabled + invert rule

;-------------------------------------------------------------

; Store cleaned data byte (with parity bit removed)

;-------------------------------------------------------------

STORE_BYTE:

MOV     @R1, A             ; Save received data to memory buffer

INC     R1                 ; Move pointer to next storage location

DEC     R0                 ; Decrement remaining receive count

MOV     A, R0

JNZ     RECEIVE_BYTE       ; If more bytes expected → continue receiving

CLR     REN                ; Disable receiver after reception finished

;===============================================================================

; PREPARE FOR TRANSMISSION

;===============================================================================

MOV     R1, #47H           ; Reset buffer pointer to start of received data

MOV     R0, #51H           ; Address for termination byte

MOV     @R0, #00H          ; Write terminating zero (optional protocol)

MOV     R0, #0BH           ; Number of bytes to transmit (11 bytes)

;===============================================================================

; TRANSMIT LOOP

; Recreate parity bit in ACC.7 before sending each byte

;===============================================================================

TRANSMIT_BYTE:

MOV     A, @R1             ; Load next data byte

INC     R1                 ; Advance buffer pointer

;-------------------------------------------------------------

; PARITY GENERATION BEFORE SENDING

;

; If B.4 = 0 → parity disabled → skip all parity logic

; If B.4 = 1 → parity enabled:

;       - CPU flag P = parity of A (auto-computed)

;       - C = P or inverted P depending on B.0

;       - ACC.7 = C → insert parity bit into MSB for transmission

;-------------------------------------------------------------

JNB     B.4, SEND_BYTE     ; If parity disabled → skip parity logic

MOV     C, P               ; C = parity of A (CPU-calculated)

JNB     B.0, SET_PARITY    ; If B.0=0 → use C as-is

CPL     C                  ; Else invert parity (different parity scheme)

SET_PARITY:

MOV     ACC.7, C           ; Insert parity into MSB

;-------------------------------------------------------------

; UART TRANSMISSION

;-------------------------------------------------------------

SEND_BYTE:

MOV     SBUF, A            ; Start UART transmission

JNB     TI, $              ; Wait until transmission completes (TI=1)

CLR     TI                 ; Clear TI for the next byte

DEC     R0                 ; Count transmitted bytes

MOV     A, R0

JNZ     TRANSMIT_BYTE      ; Continue until all bytes are sent

;===============================================================================

; ENDLESS LOOP (PROGRAM END)

;===============================================================================

END_PROGRAM:

SJMP    END_PROGRAM        ; Stop program here forever

Результат работы программы:

Ввод и вывод 10 символов (123qwe456!) в режиме без паритета

Ввод и вывод 10 символов (qwe123rty&) в режиме чётного паритета

Ввод и вывод 10 символов (Hi,Error!!) в режиме нечётного паритета

Вывод

В ходе выполнения лабораторной работы были изучены методов ввода-вывода данных через последовательный порт.