Министерство науки и высшего образования Российской Федерации

Пензенский государственный университет

Кафедра «Вычислительная техника»

ОТЧЁТ

по лабораторной работе № 9

по курсу «Проектирование программного обеспечения микропроцессорных систем»

на тему «Вывод данных через LCD - модуль»

Вариант №7

Выполнили:

Студенты группы 22ВВП1

Беляев Д. И.

Демин М. С.

Сергунов М. Р.

Приняли:

Бычков А. С.

Никишин К.И.

Пенза 2025

Цель работы

Изучение методов вывода данных через LCD-модуль.

Ход работы

Разработали программу для вывода на LCD модуль

Листинг программы:

;===============================================================================

; UART INITIALIZATION — MODE 1 (8-BIT UART, VARIABLE BAUD RATE)

;===============================================================================

CLR     SM0                ; Select UART Mode 1: SM0=0, SM1=1

SETB    SM1                ; Mode 1 → 1 start bit, 8 data bits, 1 stop bit

;-------------------------------------------------------------------------------

; Enable Double Baud Rate (PCON.7 = SMOD = 1)

;-------------------------------------------------------------------------------

MOV     A, PCON            ; Preserve other PCON bits

SETB    ACC.7              ; Set SMOD=1 → double UART baud rate

MOV     PCON, A            ; Write modified value back

;-------------------------------------------------------------------------------

; Enable UART Receiver

;-------------------------------------------------------------------------------

SETB    REN                ; Enable UART reception (REN=1)

;===============================================================================

; TIMER1 CONFIGURATION FOR UART BAUD RATE GENERATION

; Timer1 → Mode 2 (8-bit auto-reload)

; Reload value = 243 → 13-cycle reload (frequency depends on Fosc and SMOD)

;===============================================================================

MOV     TMOD, #20H         ; Timer1 Mode 2, Timer0 unchanged

MOV     TH1,  #243         ; Auto-reload value

MOV     TL1,  #243         ; Initial timer value

SETB    TR1                ; Start Timer1

;===============================================================================

; UART RECEIVE BUFFER INITIALIZATION

; R1 → pointer to RAM buffer

; R0 → remaining number of bytes to receive

;===============================================================================

MOV     R1, #47H           ; Base address of receive buffer

MOV     R0, #0AH           ; Expect to receive 10 bytes

;===============================================================================

; UART RECEIVE LOOP

; Each received byte may include a parity flag inside bit7

;===============================================================================

RECEIVE_BYTE:

JNB     RI, $              ; Wait until RI=1 (byte received)

CLR     RI                 ; Must clear RI manually

MOV     A, SBUF            ; Fetch received byte

;-------------------------------------------------------------------------------

; PARITY PROCESSING

; Bit7 of received byte may indicate presence of parity bit.

; If ACC.7 = 1 → parity information included

;

; Steps:

;   1) Clear ACC.7 to restore original 7-bit/8-bit data

;   2) CPU automatically updates flag P with parity of ACC

;   3) Store parity mode flags into register B:

;         B.4 = 1 → parity used

;         B.0 = 0/1 → parity type (even/odd)

;-------------------------------------------------------------------------------

JNB     ACC.7, STORE_BYTE  ; If bit7=0 → no parity → skip

CLR     ACC.7              ; Remove parity flag bit (updates P flag)

JNB     P, PARITY_EVEN     ; If P=0 → even number of 1s

; Odd parity detected

MOV     B, #10H            ; B = 0001 0000b → parity enabled, odd

SJMP    STORE_BYTE

PARITY_EVEN:

MOV     B, #11H            ; B = 0001 0001b → parity enabled, even

;-------------------------------------------------------------------------------

; STORE RECEIVED BYTE (parity removed)

;-------------------------------------------------------------------------------

STORE_BYTE:

MOV     @R1, A             ; Store data to buffer

INC     R1                 ; Move to next location

DEC     R0                 ; Decrease remaining count

MOV     A, R0

JNZ     RECEIVE_BYTE       ; Receive next byte

CLR     REN                ; Disable receiver when done

;===============================================================================

; RESET DATA POINTER

;===============================================================================

MOV R1, #47H               ; Reset pointer to start of received data

;===============================================================================

; LCD INITIALIZATION (4-bit INTERFACE MODE, HD44780 COMPATIBLE)

; RS = P1.3, E = P1.2, Data (DB7..DB4) = P1.7..P1.4

; All commands are sent as two nibbles (high nibble first)

;===============================================================================

CLR P1.3                   ; RS=0 → command mode

;===============================================================================

; FUNCTION SET — ENTER 4-BIT MODE (sent as 3 separate high-nibble writes)

; Sequence required by HD44780 spec (see datasheet pp. 39–42)

;===============================================================================

;--- First high nibble: 0010b → request 4-bit mode ---------------------

CLR P1.7                  ; DB7=0

CLR P1.6                  ; DB6=0

SETB P1.5                 ; DB5=1

CLR P1.4                  ; DB4=0

SETB P1.2                 ; E=1

CLR  P1.2                 ; E=0 → latch nibble

CALL delay

;--- Second identical high nibble: required by initialization protocol --

SETB P1.2

CLR  P1.2

;--- Third nibble (low part of function set): N=1, F=0 -----------------

SETB P1.7                 ; DB3=1 (N=1 → 2 lines)

; DB2..DB0 kept =0         (F=0 → 5x8 font)

SETB P1.2

CLR  P1.2

CALL delay

;===============================================================================

; ENTRY MODE SET — CURSOR INCREMENT, NO DISPLAY SHIFT

; Command: 0000 0110b

;===============================================================================

; High nibble = 0000

CLR P1.7

CLR P1.6

CLR P1.5

CLR P1.4

SETB P1.2

CLR  P1.2

; Low nibble = 0110 (I/D=1 → increment, S=0 → no shift)

SETB P1.7                 ; DB3=1 → identifies command type

SETB P1.6                 ; DB2=1 → increment cursor

; DB1=0 (S=0)

; DB0=0

SETB P1.2

CLR  P1.2

CALL delay

;===============================================================================

; DISPLAY CONTROL — DISPLAY ON, CURSOR ON, BLINK ON

; Command: 0000 1111b

;===============================================================================

; High nibble = 0000

CLR P1.7

CLR P1.6

CLR P1.5

CLR P1.4

SETB P1.2

CLR  P1.2

; Low nibble = 1111 (D=1, C=1, B=1)

SETB P1.7                 ; DB3=1

SETB P1.6                 ; DB2=1 → display ON

SETB P1.5                 ; DB1=1 → cursor ON

SETB P1.4                 ; DB0=1 → blink ON

SETB P1.2

CLR  P1.2

CALL delay

;===============================================================================

; CURSOR/DISPLAY SHIFT — CURSOR MOVE RIGHT

; Command: 0001 0110b (cursor shift to the right)

;===============================================================================

; High nibble = 0000

CLR P1.7

CLR P1.6

CLR P1.5

CLR P1.4

SETB P1.2

CLR  P1.2

; Low nibble = 0110 (S/C=0 → move cursor, R/L=1 → right)

CLR  P1.7                ; DB3=0 → cursor shift, not display shift

SETB P1.6                ; DB2=1 → shift right

SETB P1.5                ; DB1=1 (don’t care)

CLR  P1.4                ; DB0=0

SETB P1.2

CLR  P1.2

CALL delay

;===============================================================================

; SEND RECEIVED BUFFER TO LCD (RS=1 → data mode)

;===============================================================================

SETB P1.3                 ; RS=1 → write character data

MOV  R1, #47H             ; Pointer to ASCII buffer

DISPLAY_LOOP:

MOV     A, @R1            ; Load byte from buffer

JZ      FINISH            ; 0x00 → end of string

CALL    sendCharacter     ; Send ASCII to LCD

INC     R1

JMP     DISPLAY_LOOP

FINISH:

JMP     $                 ; Halt

;===============================================================================

; sendCharacter — SEND ONE ASCII BYTE IN 4-BIT MODE

; Sends high nibble first, then low nibble

;===============================================================================

sendCharacter:

;--- High nibble (ACC.7..ACC.4) ----------------------------------------

MOV C, ACC.7

MOV P1.7, C

MOV C, ACC.6

MOV P1.6, C

MOV C, ACC.5

MOV P1.5, C

MOV C, ACC.4

MOV P1.4, C

SETB P1.2

CLR  P1.2                 ; latch high nibble

;--- Low nibble (ACC.3..ACC.0) -----------------------------------------

MOV C, ACC.3

MOV P1.7, C

MOV C, ACC.2

MOV P1.6, C

MOV C, ACC.1

MOV P1.5, C

MOV C, ACC.0

MOV P1.4, C

SETB P1.2

CLR  P1.2                 ; latch low nibble

CALL delay

RET

;===============================================================================

; delay — SHORT SOFTWARE DELAY

;===============================================================================

delay:

MOV R0, #50

DJNZ R0, $

RET

Результат работы программы

Ввод символов через UART

Занесение символов в память ASCII кодом и вывод значений на LCD модуль

Вывод

В ходе выполнения лабораторной работы были изучены методы вывода данных через LCD-модуль.